Technical Requirements & Analysis WhatsApp IoT Smart Home Control System IPX800 V4 + Meta WhatsApp Cloud API + Node.js + Python FastAPI + DDNS + Port Forwarding

| Document Version | v1.0 |
| --- | --- |
| Date | May 2026 |
| Status | Updated — As-Built (reflects deployed implementation) |
| Classification | Internal Technical |
| Hardware | GCE Electronics IPX800 V4 |
| Interfaces | Meta WhatsApp Cloud API + Node.js + Python |

Abstract: This document consolidates all requirements gathered across the full design conversation, combined with analysis of the IPX800 V4 hardware manual. UPDATED: This version reflects the as-built implementation. Key divergence from the original design: the VPS and IPX800 are on the same LAN -- no DDNS, no Cloudflare Tunnel, and no router port forwarding are used. The authoritative as-built reference is docs/documentation.md.

# Table of Contents

# 1. Project Background & Objectives

## 1.1 Origin of Requirements

This system was conceived through a series of design discussions covering hardware selection, network topology decisions, API options, and synchronization strategy. The requirements below are distilled from those conversations.

## 1.2 What the System Must Do

At the highest level, the system has three missions:

| Mission | Description | Example |
| --- | --- | --- |
| M1 | Control physical devices via WhatsApp text | User types 'light on' → relay closes → light turns on |
| M2 | Report accurate real-time device status | User types 'status' → bot queries IPX800 → reports actual relay states |
| M3 | Detect and notify physical state changes | Wall switch flipped → IPX800 push → bot sends WhatsApp alert |

## 1.3 User Interaction Model

WhatsApp serves as the sole user interface. There is no dedicated app, no website, no touchscreen. The user opens a standard WhatsApp chat with the bot number and types plain text commands. The bot replies with confirmation or status information in the same chat thread.

NOTE Why WhatsApp? No additional app to install — WhatsApp is already on every phone. Works from anywhere in the world as long as the phone has internet access. Conversation history provides a natural audit log of all actions taken. Multi-user support is straightforward — any authorized number can send commands.

# 2. Functional Requirements

## 2.1 Command & Control Requirements

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| FR-01 | Light control | Bot must turn a specific light ON or OFF when commanded by an authorized user |
| FR-02 | Curtain UP control | Bot must activate the curtain-up relay while simultaneously ensuring the curtain-down relay is OFF (interlock) |
| FR-03 | Curtain DOWN control | Bot must activate the curtain-down relay while simultaneously ensuring the curtain-up relay is OFF (interlock) |
| FR-04 | Curtain STOP | Bot must deactivate both curtain relays simultaneously |
| FR-05 | Multi-device naming | Commands must support device identification by name (e.g. 'living room light') or by number (e.g. '1 on') |
| FR-06 | Command confirmation | After every successful relay operation, bot must send a WhatsApp reply confirming the action taken |
| FR-07 | Error notification | If a command fails (IPX800 unreachable, timeout, wrong command), bot must reply with a meaningful error message |
| FR-08 | Help command | User can type 'help' to receive a full list of available commands and device names |

## 2.2 Status & Synchronization Requirements

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| FR-09 | Real-time status query | When user sends 'status', bot must query IPX800 status.xml and return the actual current state of all devices |
| FR-10 | Physical change detection | When a physical wall switch changes a relay state, the system must detect this change via IPX800 push notification |
| FR-11 | Physical change alert | Upon detecting a physical state change, bot must proactively send a WhatsApp alert to the admin number specifying which device changed, to what state, and at what time |
| FR-12 | State cache | System must maintain an in-memory state cache recording the last known status, source (whatsapp/physical), and timestamp for each device |
| FR-13 | Cache accuracy | The state cache must be updated after every command execution AND after every IPX800 push notification received |
| FR-14 | Power cut recovery | If IPX800 reboots after a power cut, it must push the new state of all relays so the bot can update its cache and alert the user |

## 2.3 Security Requirements

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| FR-15 | Authorized number whitelist | Only phone numbers listed in the configuration file may send commands. All other numbers are silently ignored |
| FR-16 | Webhook signature verification | Every incoming Webhook POST from Meta must be verified using HMAC SHA256 before any processing occurs |
| FR-17 | Message deduplication | The system must deduplicate incoming messages using the message ID to prevent double execution caused by Meta retries |
| FR-18 | APIKEY protection | The IPX800 APIKEY must never be hardcoded in source code. It must be stored in an environment variable file with restricted permissions (chmod 600) |
| FR-19 | HTTPS everywhere | All external-facing endpoints must use TLS. Meta Webhook requires HTTPS. IPX800 push to VPS must use SSL (TLS 1.0 supported by IPX800 V4) |

## 2.4 Reliability Requirements

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| FR-20 | Webhook response time | The Webhook endpoint must return HTTP 200 to Meta within 20 seconds. IPX800 control must be executed asynchronously after returning 200 |
| FR-21 | Retry on IPX800 failure | If IPX800 is unreachable, the system must retry up to 3 times with 10-second intervals before reporting failure to the user |
| FR-22 | Process auto-restart | The backend service must automatically restart after a crash using systemd with Restart=always |
| FR-23 | Certificate auto-renewal | The Let's Encrypt TLS certificate must auto-renew and the service must restart automatically after renewal |
| FR-24 | Curtain hardware interlock | The curtain UP relay and DOWN relay must never be activated simultaneously. Software must enforce a minimum 200ms gap between deactivating one and activating the other |

# 3. Hardware Analysis — IPX800 V4

## 3.1 Hardware Specifications (from Manual v4.04.05)

| Specification | Value |
| --- | --- |
| Manufacturer | GCE Electronics (Made in France) |
| Model | IPX800 V4 — Ethernet Control System |
| Relay outputs | 8 relays (6 with changeover contact), 270V / 10A, expandable to 56 |
| Digital inputs | 8 built-in dry contact inputs (active low), expandable to 56 |
| Analog inputs | 4 inputs (0–3.3V / 10 bits) |
| Push notifications | 128 push presets — HTTP GET or POST to any URL, with optional TLS 1.0 |
| Scenario engine | 256 programmable scenarios: event (AND/OR/NOT logic) → action (ON/OFF/SWITCH) → result (relay/push/email) |
| API type | HTTP REST, protected by APIKEY (default key name: apikey) |
| API M2M port | 9870 (TCP/IP M2M, configurable) |
| HTTP port | 80 (default, configurable) |
| Relay memory mode | Optional — preserves relay states across power cuts |
| Relay timing | Ta (pre-delay) and Tb (auto-off timer) configurable per relay |
| DynDNS | Built-in support |
| Network | 10/100 Mbps, Energy Efficient Ethernet (IEEE 802.3az) |
| Power supply | 10–18V DC, 0.9W idle / 3.5W all relays active |
| Operating temperature | -10 to +60°C at 50% humidity |
| Form factor | DIN rail mount, 9 DIN slots |

## 3.2 Relay Control API

The IPX800 V4 exposes a simple HTTP REST interface. All commands are GET requests with the APIKEY passed as a URL parameter.

Relay activation/deactivation:

GET http://{IPX800_IP}/preset.htm?led{N}={0|1}&apikey={APIKEY}

Parameters:

- N: relay number 1–8 (note: status.xml uses 0-based index led0 for relay 1)

- state: 1 = close (activate), 0 = open (deactivate)

- apikey: the key configured in the IPX800 Network > API settings page

Status query — returns XML with all relay states:

GET http://{IPX800_IP}/status.xml?apikey={APIKEY}

Response structure:

<response>

<led0>0</led0>   <!-- Relay 1: 0=OFF, 1=ON (0-based index!) -->

<led1>1</led1>   <!-- Relay 2 -->

<led2>0</led2>   <!-- Relay 3 -->

...

</response>

WARNING Critical: 0-Based Index in status.xml The relay control URL uses 1-based numbering: preset.htm?led1=1 controls Relay 1. But status.xml uses 0-based numbering: <led0> corresponds to Relay 1. This discrepancy must be handled carefully in code to avoid controlling the wrong relay.

## 3.3 Push Notification System (Physical Sync Mechanism)

The IPX800 V4 includes a built-in push notification engine that can send HTTP requests to any URL when a scenario is triggered. This is the mechanism used to notify the VPS when a physical wall switch changes a relay state.

### 3.3.1 How Push Works

- Up to 128 push presets can be configured in the IPX800 Web UI under the Push section

- Each preset defines: server address, port, URL ON (sent when event=1), URL OFF (sent when event=0), method (GET or POST), and optional SSL (TLS 1.0)

- Push presets are triggered by Scenario results — not directly by relay changes

- A Scenario links an event source (e.g. Digital Input 1) to an action type (ON/OFF) and a result (push preset)

- The URL fields support variable substitution using the $ character to embed relay states or other values

### 3.3.2 Push Configuration for Physical Sync

For each device that can be physically switched, configure one push preset and one scenario:

| Push Field | Light Example | Curtain UP Example |
| --- | --- | --- |
| Preset Name | Light Physical Change | Curtain UP Physical Change |
| Server | yourdomain.com | yourdomain.com |
| Port | 443 | 443 |
| URL ON | /ipx-notify?device=light&state=1 | /ipx-notify?device=curtain_up&state=1 |
| URL OFF | /ipx-notify?device=light&state=0 | /ipx-notify?device=curtain_up&state=0 |
| Method | GET | GET |
| SSL | ON (TLS 1.0) | ON (TLS 1.0) |

### 3.3.3 Scenario Configuration for Physical Sync

| Scenario Field | Configuration |
| --- | --- |
| Scenario Name | Light Switch Detection |
| Event type | Digital Input (IPX800 V4) |
| Event source | Digital Input 1 (wired to wall switch) |
| Logic | Simple — no AND/OR needed |
| Action type | ON/OFF — fires URL ON when input=1, URL OFF when input=0 |
| Result type | PUSH — selects the corresponding push preset |

NOTE Digital Input Wiring Requirement For physical sync to work, the wall switch must be wired to an IPX800 V4 Digital Input terminal (DI1-DI8). IPX800 digital inputs work on dry contact to ground (active low). If the wall switch only controls the relay directly via the electrical circuit (not wired to a DI), the IPX800 cannot detect the state change. This is an open hardware question that must be confirmed before implementing physical sync.

## 3.4 Relay Memory Mode

The IPX800 V4 supports an optional memory mode per relay. When enabled, the relay restores its last state after a power cut rather than defaulting to OFF. This is critical for lights that the user expects to remain on after a brief power interruption.

Configuration: IPX800 Web UI > IPX800V4 > Relay Outputs > Memory Mode toggle per relay.

## 3.5 Relay Timing Parameters

Each relay has two configurable timing parameters that may be relevant for curtain control:

- Ta (pre-delay): delays the relay activation by a configurable number of seconds after the command is received

- Tb (auto-off timer): automatically deactivates the relay after a set number of seconds — useful for timed curtain movement (e.g. open 50% = run motor for 15s then stop)

# 4. System Architecture

## 4.1 Confirmed Network Topology

The following topology reflects the as-built deployment: VPS and IPX800 are on the same LAN. The FastAPI service reaches the IPX800 directly by its local IP address -- no DDNS, no port forwarding, no router traversal is required.

| Component | Location | Network Role |
| --- | --- | --- |
| VPS (Ubuntu) | Cloud provider (OVH/Hetzner) | Public IP + domain name. Hosts both backend services (Node.js + Python FastAPI). Reaches IPX800 directly via shared LAN -- no DDNS resolution needed. |
| Home Router | Home LAN gateway | VPS and IPX800 share the same LAN. No port forwarding configured. IPX800 assigned a static LAN IP in its Web UI. |
| IPX800 V4 | Home LAN | Static LAN IP (e.g. 192.168.1.x, set in IPX800 Web UI). Reachable directly from VPS: http://<IPX800-LAN-IP>:80 -- no external exposure. |
| Meta Cloud API | Meta global CDN | Pushes Webhook POSTs to VPS over public internet (HTTPS) |
| WhatsApp User | Anywhere (mobile phone) | Sends/receives messages via Meta infrastructure |

WARNING Critical Pre-condition: Verify Real Public IP (Not CG-NAT) Before building anything, run this command on any device connected to the home network: curl ifconfig.me Then check the WAN IP shown in your router admin page. If both match -> you have a real public IP -> Option A (DDNS + port forwarding) will work. If they differ -> you are behind CG-NAT -> port forwarding is impossible. Escalate to Option B (frp reverse tunnel).

## 4.2 Full System Data Flow

// ── Complete flow for a WhatsApp command ─────────────────────────────────

User sends 'light on' via WhatsApp

|

v  HTTPS POST

Meta Cloud API

|  POST https://yourdomain.com/webhook

|  Header: X-Hub-Signature-256: sha256=<hmac>

v

Node.js Express /webhook (VPS port 443)

|  1. Verify HMAC SHA256 signature

|  2. Parse message body -- extract sender number + text

|  3. Check authorized number whitelist

|  4. Deduplicate by message ID

|  5. Return HTTP 200 to Meta immediately  <-- critical, within 20s

|  6. BackgroundTask: call Python FastAPI /control

v

Python FastAPI /control

|  7. Map command to relay number

|  8. Apply curtain interlock if needed (200ms gap)

|  9. GET http://<IPX800-LAN-IP>:80/preset.htm?led1=1&apikey=XXX

|     (direct LAN -- VPS and IPX800 on same network, no DDNS needed)

|  10. Read back /status.xml to confirm execution

|  12. Update state cache (status=ON, source=whatsapp, time=now)

v

Node.js sends WhatsApp reply

|  POST https://graph.facebook.com/v19.0/{PHONE_ID}/messages

v

User receives: 'Light turned ON'

// ── Physical switch flow ─────────────────────────────────────────────────

Physical wall switch changes state

|  IPX800 Digital Input detects transition

v

IPX800 Scenario fires

|  Scenario: DI1 change -> PUSH preset

v

IPX800 sends GET https://yourdomain.com/ipx-notify?device=light&state=0

|  IPX800 resolves yourdomain.com -> VPS public IP

|  Push uses SSL (TLS 1.0 supported by IPX800 V4) on port 443

v

Node.js /ipx-notify endpoint (VPS)

|  Update state cache (status=OFF, source=physical, time=now)

|  Send WhatsApp alert to admin number

v

User receives: 'Light was physically switched OFF at 14:32'

# 5. Complete Technology Stack

## 5.1 Layer-by-Layer Stack Overview

| Layer | Technology | Version | Role |
| --- | --- | --- | --- |
| Hardware | GCE IPX800 V4 | Firmware 4.04.05 | 8-relay Ethernet controller, physical I/O |
| OS | Ubuntu | 22.04 LTS | VPS operating system |
| Runtime A | Node.js | 20 LTS | Webhook receiver + WhatsApp sender |
| Framework A | Express.js | 4.x | HTTP routing for Webhook and push endpoints |
| Runtime B | Python | 3.10+ | IPX800 controller + state cache + command logic |
| Framework B | FastAPI | 0.110+ | Async REST API, BackgroundTasks, internal control API |
| ASGI Server | Uvicorn | 0.29+ | Serves FastAPI with direct SSL termination |
| HTTP Client | httpx (Python) | 0.27+ | Async HTTP calls to IPX800 via local LAN IP address (direct, no DDNS) |
| HTTP Client | https (Node.js) | Built-in | WhatsApp Graph API calls |
| Config | python-dotenv | 1.0+ | Load .env variables in Python |
| Config | dotenv (npm) | 16+ | Load .env variables in Node.js |
| TLS Certificate | Let's Encrypt | certbot | Free 90-day auto-renewable HTTPS cert |
| Process Manager | systemd | System | Auto-restart on crash, boot startup |
| Message API | Meta WhatsApp Cloud API | v19.0 | Send and receive WhatsApp messages |
| Security | HMAC SHA256 | Built-in | Webhook signature verification |

## 5.2 Node.js Service — Webhook & Messaging

The Node.js Express service handles everything related to WhatsApp communication. It is the public-facing component that Meta reaches via HTTPS.

| Component | Details |
| --- | --- |
| Entry point | app.js — Express server, SSL via Node https module, port 443 |
| GET /webhook | Meta handshake verification — checks hub.verify_token, returns hub.challenge |
| POST /webhook | Receives WhatsApp messages — verifies HMAC, parses JSON, checks whitelist, deduplicates, calls FastAPI asynchronously |
| GET /ipx-notify | Receives IPX800 push notifications — updates state cache, sends WhatsApp alert |
| GET /health | Service health check — returns uptime and last IPX800 contact time |
| whatsapp.js | Sends messages via Meta Graph API (POST /v19.0/{PHONE_ID}/messages) |
| state.js | In-memory state cache for all devices with status, source, and timestamp |
| security.js | HMAC SHA256 verification, whitelist check, message ID deduplication |
| Environment | PORT, VERIFY_TOKEN, WA_PHONE_NUMBER_ID, WA_ACCESS_TOKEN, WA_APP_SECRET, ALLOWED_NUMBERS |

## 5.3 Python FastAPI Service — Device Control

The Python FastAPI service handles all hardware interaction. It is an internal service called by Node.js. It reaches the IPX800 directly via its local LAN IP address -- no DDNS or port forwarding needed since VPS and IPX800 share the same LAN.

| Component | Details |
| --- | --- |
| Entry point | main.py — FastAPI app, internal port (e.g. 8000), VPS-internal only (not public-facing) |
| POST /control | Receives command from Node.js, maps to relay number, calls IPX800 via local LAN IP, applies curtain interlock if needed, returns reply text and success flag |
| GET /health | Pings IPX800 to check reachability via local LAN IP; returns structured JSON health report with ipx800_host, ipx800_port, and reachability status |
| ipx800.py | IPX800 HTTP client — relay control, status read, interlock logic, retry on failure, handles DDNS lag |
| commands.py | Keyword mapping — maps text commands to relay actions (supports English + French keywords) |
| config.py | Loads IPX800_HOST (<IPX800-LAN-IP>), IPX800_PORT (80), IPX800_APIKEY, relay mapping (RELAY_LIGHT, RELAY_CURTAIN_UP, RELAY_CURTAIN_DOWN), and log settings from fastapi/.env |
| Curtain interlock | set_relay(DOWN, OFF) → asyncio.sleep(0.2) → set_relay(UP, ON) — enforced in ipx800.py |
| Retry logic | 3 attempts with 10 s delay -- covers transient LAN errors. After all retries exhausted, returns success:false to Node.js which sends an error reply to the user. |

## 5.4 Meta WhatsApp Cloud API Integration

| Parameter | Value / Notes |
| --- | --- |
| API Version | v19.0 (use latest stable) |
| App Type | Business (must be Business, not Consumer) |
| Token Type | System User Token — Never expires. Generated in Meta Business Settings > Users > System Users |
| Required Permissions | whatsapp_business_messaging + whatsapp_business_management |
| Webhook Events | Subscribe to: messages |
| Webhook Verification | hub.mode=subscribe + hub.verify_token (custom string stored in .env) |
| Signature Header | X-Hub-Signature-256: sha256=<HMAC-SHA256 of body using App Secret> |
| Send Message Endpoint | POST https://graph.facebook.com/v19.0/{PHONE_NUMBER_ID}/messages |
| Testing Limit | Up to 5 whitelisted numbers in Development mode |
| 24h CSW | Free-form replies (non-template) are only allowed within the 24-hour Customer Service Window after the user sends a message first |
| Credentials needed | PHONE_NUMBER_ID, ACCESS_TOKEN (System User), APP_SECRET, VERIFY_TOKEN, WABA_ID |

# 6. Prerequisites

## 6.1 Infrastructure Prerequisites

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| P-01 | VPS and IPX800 on the same LAN -- static IP on IPX800 | Verify both are on the same network segment. Assign IPX800 a static LAN IP in its Web UI (Network > Parameters > disable DHCP). No DDNS or port forwarding needed. |
| P-02 | VPS public domain name | A domain (e.g. yourdomain.com) with DNS A record pointing to VPS public IP. Required for Meta Webhook (must be HTTPS). Note: this is different from the DuckDNS hostname used for IPX800. |
| P-03 | Port 443 open on VPS firewall | ufw allow 443. Meta Webhook only accepts HTTPS on standard port 443. |
| P-04 | Let's Encrypt certificate for VPS domain | certbot certonly --standalone -d yourdomain.com. Auto-renewal configured with deploy-hook to restart services. |

## 6.2 Hardware Prerequisites

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| P-06 | Relay wiring | Relay 1 → light circuit, Relay 2 → curtain motor UP, Relay 3 → curtain motor DOWN. Wiring must comply with 270V/10A rating. |
| P-07 | Curtain motor compatibility | Curtain motor must be a reversible type that accepts UP/DOWN relay control. Confirm motor voltage and current draw within IPX800 relay spec. |
| P-08 | Digital input wiring (for physical sync) | Wall switches must be wired to IPX800 Digital Inputs (DI1-DI8) using dry contact to GND. Without this, physical state changes cannot be detected by the scenario engine. |
| P-09 | IPX800 power supply | 12V DC adapter (not included). Stable power supply required for reliable relay operation and LAN connectivity. |
| P-10 | IPX800 APIKEY configured | In IPX800 Web UI: Network > API > enable API key protection > set APIKEY value. Same key stored in .env as IPX800_APIKEY. |

## 6.3 Meta Account Prerequisites

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| P-11 | Meta Developer account | Register at developers.facebook.com. Free. Required to create the Business App. |
| P-12 | Meta Business Portfolio | Business account at business.facebook.com. Required to attach to the WhatsApp product. |
| P-13 | Business App created | App type must be Business. WhatsApp product added. App ID recorded. |
| P-14 | Test phone number whitelisted | Up to 5 numbers added to test whitelist in API Setup page. Each must verify via OTP. |
| P-15 | System User Token generated | In Business Settings > Users > System Users. Token expiration set to Never. Permissions: whatsapp_business_messaging + whatsapp_business_management. |
| P-16 | App Secret recorded | Found in App Dashboard > Settings > Basic > App Secret. Used for Webhook HMAC verification. |
| P-17 | Webhook registered | Callback URL: https://yourdomain.com/webhook. Verify Token matches .env value. messages event subscribed. |

## 6.4 Software Prerequisites

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| P-18 | Node.js 20 LTS | Install on VPS: curl -fsSL https://deb.nodesource.com/setup_20.x ¦ sudo bash - && sudo apt install nodejs |
| P-19 | Python 3.10+ | Install on VPS: sudo apt install python3.10 python3-pip python3-venv |
| P-20 | npm packages | express, dotenv. Install in project directory: npm install express dotenv |
| P-21 | pip packages | fastapi, uvicorn[standard], httpx, python-dotenv. Install: pip install -r requirements.txt |
| P-22 | certbot | sudo apt install certbot. Run: certbot certonly --standalone -d yourdomain.com |
| P-23 | systemd service files | One .service file per process (Node.js + Python FastAPI). Enable with systemctl enable + start. |
| P-24 | .env file | Created at /opt/home-control/.env with chmod 600. Contains all secrets and configuration. Not committed to git. |

# 7. Command & Device Design

## 7.1 Relay Mapping

| Relay | Device | IPX800 led# (control) | led# in status.xml |
| --- | --- | --- | --- |
| Relay 1 | Light | led1=1 (ON) / led1=0 (OFF) | <led0> |
| Relay 2 | Curtain UP | led2=1 (activate) / led2=0 (deactivate) | <led1> |
| Relay 3 | Curtain DOWN | led3=1 (activate) / led3=0 (deactivate) | <led2> |

## 7.2 Full Command Reference

| User Types | Also Accepts | Relay Action | Bot Reply |
| --- | --- | --- | --- |
| light on | 1 on | Relay1=ON | Light turned ON |
| light off | 1 off | Relay1=OFF | Light turned OFF |
| curtain up | 2 up / open curtain | Relay2=ON + Relay3=OFF (with 200ms gap) | Curtain moving up |
| curtain down | 2 down / close curtain | Relay2=OFF + Relay3=ON (with 200ms gap) | Curtain moving down |
| curtain stop | 2 stop / stop curtain | Relay2=OFF + Relay3=OFF | Curtain stopped |
| status | s | Query status.xml | Light: ON ¦ Curtain: Stopped |
| help | h / ? | No relay action | Full command list displayed |

## 7.3 Status Report Format

When the user sends 'status', the bot replies with a structured report:

Home Status — 14:32:05

Light:          ON  (via WhatsApp at 14:30)

Curtain:     Stopped  (physical switch at 14:25)

All devices confirmed from IPX800.

# 8. Security Design

| Threat | Risk | Protection | Implementation |
| --- | --- | --- | --- |
| Forged Webhook requests | HIGH | HMAC SHA256 signature | Verify X-Hub-Signature-256 header on every POST using App Secret |
| Unauthorized WhatsApp control | HIGH | Number whitelist | ALLOWED_NUMBERS in .env, checked before any command is processed |
| IPX800 APIKEY exposure | HIGH | Environment variable | Stored in .env (chmod 600), never in source code, never logged in full |
| Man-in-the-middle (Webhook) | HIGH | TLS (Let's Encrypt) | HTTPS on port 443, cert auto-renewed, HSTS header recommended |
| IPX800 unauthorized access | MEDIUM | LAN isolation | IPX800 on private LAN, not reachable from public internet |
| Meta Access Token expiry | HIGH | System User Token | Token set to Never expire; monitor for API 401 errors |
| Webhook duplicate delivery | MEDIUM | Message ID cache | Deque of last 200 message IDs; duplicates silently discarded |
| Curtain motor damage | HIGH | Software interlock | 200ms enforced gap between relay transitions in ipx800.py |
| Service crash | MEDIUM | systemd Restart=always | RestartSec=3, automatic recovery within seconds |
| Log file disk overflow | LOW | Log rotation | RotatingFileHandler, 10MB x 7 files max |
| Brute force scanning | MEDIUM | fail2ban | Auto-ban IPs with repeated 403 responses on port 443 |

# 9. Risk Register

| # | Risk | Level | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| R1 | Wall switch NOT wired to Digital Input | HIGH | Physical sync impossible — IPX800 cannot detect switch state | Confirm wiring before building sync module. Fallback: polling every 60s |
| R2 | Curtain interlock failure — motor damaged | HIGH | Physical hardware damage | Code-level AND hardware-level interlock. Test with unpowered motor first |
| R3 | Meta Access Token expires | HIGH | WhatsApp replies stop silently | Use System User Token (never expires). Add 401 error monitoring |
| R4 | IPX800 0-based vs 1-based index confusion | HIGH | Wrong relay activated — dangerous if curtain relays swapped | Unit test relay mapping with physical verification before any live testing |
| R5 | Webhook duplicate delivery | MEDIUM | Same command executed twice | message_id deduplication in security.js |
| R6 | Let's Encrypt certificate renewal interrupts service | MEDIUM | Meta Webhook delivery fails during downtime | certbot deploy-hook restarts service. Renewal window is short (seconds) |
| R7 | IPX800 reboot clears relay state (if memory mode OFF) | MEDIUM | Lights/curtains reset unexpectedly | Enable memory mode in IPX800 Web UI for relays where appropriate |
| R8 | Router port forwarding rule lost after reboot | MEDIUM | IPX800 unreachable until rule is re-applied | Set rule as permanent in router config. Test after router reboot. |
| R9 | 24-hour WhatsApp CSW closed | LOW | Bot cannot send free-form replies | Mitigation: user must send a message first. Consider template fallback |

# 10. Implementation Plan

## 10.1 Phase Overview

| # | Phase | Goal | Est. Time | Exit Criteria |
| --- | --- | --- | --- | --- |
| 1 | Network & Hardware Verification | Confirm VPS reaches IPX800 directly | 1–2h | curl from VPS returns valid status.xml |
| 2 | IPX800 API Validation | All relay operations tested via curl | 1h | All 6 relay actions physically confirmed |
| 3 | VPS Environment Setup | Node.js + Python + TLS ready | 1h | https://yourdomain.com/health returns 200 |
| 4 | Meta WhatsApp API Setup | Can send and receive WhatsApp messages | 1–2h | Test message received + replied |
| 5 | Core Backend — Commands | WhatsApp commands control relays | 3–4h | All 7 commands work end-to-end |
| 6 | Physical Sync | IPX800 push updates state + alerts WhatsApp | 2h | Flip wall switch → WhatsApp alert received |
| 7 | Security Hardening | All security measures active | 1h | Forged request returns 403, unknown number ignored |
| 8 | Deployment & Testing | Stable production service | 1h | VPS reboot → service auto-recovers |

Total estimated time: 11–14 hours across all phases.

## 10.2 Phase 1 — Network & Hardware Verification (Do This First)

These steps must be completed and confirmed before writing any code. They validate the core assumptions of the entire architecture.

| # | Action / Requirement | Details / Notes |
| --- | --- | --- |
| 1.1 | Verify VPS and IPX800 on the same LAN | Confirm both are on the same network. Ping IPX800 LAN IP from VPS. Find IPX800 LAN IP in its Web UI (Network > Parameters) or router DHCP table. |
| 1.2 | Assign IPX800 a static LAN IP | IPX800 Web UI: Network > Parameters > disable DHCP > set fixed IP (e.g. 192.168.1.100). Prevents IP change on DHCP renewal. |
| 1.3 | Set IPX800_HOST and IPX800_APIKEY in fastapi/.env | Edit fastapi/.env: set IPX800_HOST=<IPX800-LAN-IP> (e.g. 192.168.1.100), IPX800_PORT=80, and IPX800_APIKEY=<your-key>. |
| 1.4 | Set IPX800 to static LAN IP (confirm in Web UI) | IPX800 Web UI: Network > Parameters > disable DHCP > set fixed IP (e.g. 192.168.1.100) > save and reboot. |
| 1.5 | Configure APIKEY on IPX800 | Web UI: Network > API > enable API key protection > set key value > save. Use same key as IPX800_APIKEY in fastapi/.env. |
| 1.6 | Test relay control from VPS via direct LAN | From VPS: curl "http://<IPX800-LAN-IP>:80/preset.htm?led1=1&apikey=XXX". Relay 1 (light) should activate. Verify in IPX800 web UI. |
| 1.7 | Test status.xml read from VPS | From VPS: curl "http://<IPX800-LAN-IP>:80/status.xml?apikey=XXX". Should return XML with <led0>, <led1>, <led2> values. |
| 1.8 | Confirm Digital Input wiring | Verify wall switches wired to DI terminals DI1-DI8. Toggle switch, check DI state in IPX800 Web UI dashboard. |

## 10.3 Open Questions Before Development Starts

| # | Question | Why It Matters |
| --- | --- | --- |
| Q1 | Are wall switches wired to IPX800 Digital Inputs DI1-DI8? | Determines whether physical sync is possible at all. Without DI wiring, IPX800 cannot trigger scenarios. |
| Q2 | How many devices total (lights + curtains)? | Determines relay mapping, command vocabulary, and number of push presets to configure. |
| Q3 | What device names should the bot recognize? | Determines keyword list in commands.py (living room, bedroom, kitchen, etc.) |
| Q4 | Is memory mode needed on relays? | If yes, lights will stay on after a power cut. If no, they will reset to OFF on reboot. |
| Q5 | Single admin number or multiple authorized users? | Determines whitelist configuration and whether to send alerts to one or multiple numbers. |
| Q6 | Should curtain control be simple (up/down/stop) or timed (open 50%)? | Timed control requires using Relay Tb parameter in IPX800 or a timer in software. |
| Q7 | Is the IPX800 firmware on v4.04.05 or newer? | Ensures all manual-documented features (push, scenarios) behave as expected. |

# 11. Environment Configuration Reference

All secrets and configuration parameters are stored in a single .env file at /opt/home-control/.env with chmod 600 permissions. Never commit this file to git.

# ── Meta WhatsApp Cloud API ──────────────────────────────────────

WA_PHONE_NUMBER_ID=1234567890123456

WA_ACCESS_TOKEN=EAAxxxxxxxxxxxxxxxxxxxxx

WA_VERIFY_TOKEN=your-random-uuid-here

WA_APP_SECRET=abcdef1234567890abcdef1234567890

WA_WABA_ID=9876543210987654

WA_API_VERSION=v19.0

# -- IPX800 V4 (accessed via local LAN IP) ---------------------

IPX800_HOST=<IPX800-LAN-IP>     # e.g. 192.168.1.100

IPX800_PORT=80

IPX800_APIKEY=your-ipx800-apikey

IPX800_TIMEOUT=5.0

IPX800_RETRY=3

# ── Relay Mapping ────────────────────────────────────────────────

RELAY_LIGHT=1

RELAY_CURTAIN_UP=2

RELAY_CURTAIN_DOWN=3

# ── Security ─────────────────────────────────────────────────────

ALLOWED_NUMBERS=33612345678,33698765432

# ── Service Ports ────────────────────────────────────────────────

NODE_PORT=443

FASTAPI_PORT=8000

# ── Logging ──────────────────────────────────────────────────────

LOG_LEVEL=INFO

LOG_FILE=/opt/home-control/logs/app.log

LOG_MAX_BYTES=10485760

LOG_BACKUP_COUNT=7

End of Document — WhatsApp IoT Smart Home: Requirements & Technical Analysis v1.0